# DECA ST2 Questions and Answers

## Question 1 (Marks: 1)

Question: How many NOT gates are required to construct a 4-to-1 multiplexer?

Options:

0. 1

1. 2

2. 3

3. 4

Correct Answer(s):

- 2

________________________________________

## Question 2 (Marks: 1)

Question: A multiplexer can be used to implement:

Options:

0. Any Boolean function

1. Only addition operations

2. Only logic NOT function

3. Memory storage

Correct Answer(s):

- Any Boolean function

________________________________________

## Question 3 (Marks: 1)

Question: In a combinational circuit, the output at any time depends only on the --------- at that time.

Options:

0. Voltage

1. Input values

2. Intermediate values

3. Clock plus

Correct Answer(s):

- Input values

________________________________________

## Question 4 (Marks: 1)

Question: The minimum number of NAND gates required to realize an AND gate is ______.

Options:

0. 2

1. 3

2. 4

3. 6

Correct Answer(s):

- 2

________________________________________

## Question 5 (Marks: 1)

Question: Which of the following combinations cannot be grouped in a K-map?

Options:

0. 1, 2, 3, 7

1. 0, 1

2. 4, 5

3. 3, 7

Correct Answer(s):

- 1, 2, 3, 7

________________________________________

## Question 6 (Marks: 1)

Question: (X+Y′)(X+Z) can be represented by ______.

Options:

0. (X+Y`Z)

1. (Y+X`)

2. XY

3. (X+Z`)

Correct Answer(s):

- (X+Y`Z)

________________________________________

## Question 7 (Marks: 1)

Question: The minimum number of NAND gates required to realize an OR gate is ______.

Options:

0. 5

1. 3

2. 4

3. 6

Correct Answer(s):

- 3

________________________________________

## Question 8 (Marks: 1)

Question: Which logic gate can be used to build all other logic gates?

Options:

0. NAND

1. AND

2. NOT

3. OR

Correct Answer(s):

- NAND

________________________________________

## Question 9 (Marks: 1)

Question: A decoder is a combinational circuit that ______.

Options:

0. Converts binary information from n input lines to 2ⁿ unique output lines

1. Converts 2ⁿ inputs to n outputs

2. Stores binary data

3. Encodes analog signals into digital form

Correct Answer(s):

- Converts binary information from n input lines to 2ⁿ unique output lines

________________________________________

## Question 10 (Marks: 1)

Question: Four statements are given below, identify the correct one

Options:

0. XOR is universal gate

1. XNOR is basic gate

2. XOR is derived gate

3. XOR is basic gate

Correct Answer(s):

- XOR is derived gate

________________________________________

## Question 11 (Marks: 1)

Question: Which of the following can be represented for decoder?

Options:

0. Sequential circuit

1. Combinational circuit

2. Analog circuit

3. None of the mentioned

Correct Answer(s):

- Combinational circuit

________________________________________

## Question 12 (Marks: 1)

Question: A 3-input binary-to-decimal decoder will have ______ outputs.

Options:

0. 6

1. 7

2. 8

3. 9

Correct Answer(s):

- 8

________________________________________

## Question 13 (Marks: 1)

Question: A 4-to-1 multiplexer has:

Options:

0. 4 inputs, 1 output, and 2 select lines

1. 2 inputs, 4 outputs, and 2 select lines

2. 4 inputs, 2 outputs, and 1 select line

3. 1 input, 4 outputs, and 4 select lines

Correct Answer(s):

- 4 inputs, 1 output, and 2 select lines

________________________________________

## Question 14 (Marks: 1)

Question: The basic SR latch can be constructed using:

Options:

0. Two XOR gates

1. Two NOR or two NAND gates

2. One AND gate

3. Two OR gates

Correct Answer(s):

- Two NOR or two NAND gates

________________________________________

## Question 15 (Marks: 1)

Question: The D flip-flop eliminates the invalid state of the SR flip-flop by ______.

Options:

0. Using only one input

1. Using two clocks

2. Cross coupling outputs

3. Feedback from

Correct Answer(s):

- Using only one input

________________________________________

## Question 16 (Marks: 1)

Question: The output of a 2-input NOR gate is HIGH when ______.

Options:

0. Both inputs are LOW

1. Both inputs are HIGH

2. Either input is HIGH

3. Both inputs are

Correct Answer(s):

- Both inputs are LOW

________________________________________

## Question 17 (Marks: 1)

Question: When two adjacent 1’s are grouped in a K-map, the number of literals in the resulting product term is reduced by ______.

Options:

0. 0

1. 1

2. 2

3. 3

Correct Answer(s):

- 1

________________________________________

## Question 18 (Marks: 1)

Question: What is the primary functioning responsibility of a multiplexing combinational circuit?

Options:

0. Decoding the binary information

1. Generation of all minterms in an output function with OR-gate

2. Generation of selected path between multiple sources and a single destination

3. Encoding of binary information

Correct Answer(s):

- Generation of selected path between multiple sources and a single destination

________________________________________

## Question 19 (Marks: 1)

Question: Latches constructed with NOR and NAND gates tend to remain latched due to which configuration feature?

Options:

0. Low input voltages

1. Synchronous operation

2. Gate impedance

3. Cross coupling

Correct Answer(s):

- Cross coupling

________________________________________

## Question 20 (Marks: 1)

Question: Which of the following is correct for a gated D-type flip-flop?

Options:

0. The Q output is either SET or RESET as soon as the D input goes HIGH or LOW

1. The output complement follows the input when enabled

2. Only one of the inputs can be HIGH at a time

3. The output toggles if one of the inputs is held HIGH

Correct Answer(s):

- The Q output is either SET or RESET as soon as the D input goes HIGH or LOW

________________________________________

## Question 21 (Marks: 1)

Question: A NAND gate has —– inputs and —— outputs.

Options:

0. High and High

1. High and low

2. Low and low

3. None of the mentioned

Correct Answer(s):

- High and low

________________________________________

## Question 22 (Marks: 1)

Question: Don’t-care terms should be used along with present terms in ______.

Options:

0. Minterms

1. Maxterms

2. both minterms and maxterms

3. Latches

Correct Answer(s):

- both minterms and maxterms

________________________________________

## Question 23 (Marks: 1)

Question: There are ______ cells in a 4-variable K-map.

Options:

0. 12

1. 16

2. 18

3. 8

Correct Answer(s):

- 16

________________________________________

## Question 24 (Marks: 1)

Question: Each product term of a group, w’.x.y’ and w.y, represents the ____________ in that group.

Options:

0. Input

1. POS

2. Sum-of-Minterms

3. Sum of Maxterms

Correct Answer(s):

- Sum-of-Minterms

________________________________________

## Question 25 (Marks: 1)

Question: Don’t-care conditions can be used for simplifying Boolean expressions in ______.

Options:

0. Registers

1. Terms

2. K-maps

3. Latches

Correct Answer(s):

- K-maps

________________________________________

## Question 26 (Marks: 1)

Question: For instruction code 7800(H), which type of instruction is defined?

Options:

0. Memory Reference

1. Register Reference

2. Input-Outpt Refernce

3. Not specified

Correct Answer(s):

- Register Reference

________________________________________

## Question 27 (Marks: 1)

Question: What is the size of the AR (address register) in a 1024×8 memory?

Options:

0. 8

1. 10

2. 9

3. 16

Correct Answer(s):

- 10

________________________________________

## Question 28 (Marks: 1)

Question: Which register is used to store operands temporarily during arithmetic and logic operations?

Options:

0. AC

1. IR

2. PC

3. AR

Correct Answer(s):

- AC

________________________________________

## Question 29 (Marks: 1)

Question: For instruction code 1800(H), which type of instruction is defined?

Options:

0. Memory Reference

1. Register Reference

2. Input-Outpt Refernce

3. Not specified

Correct Answer(s):

- Memory Reference

________________________________________

## Question 30 (Marks: 1)

Question: In a basic computer, registers that hold data and registers that hold addresses are ______.

Options:

0. (AC,DR,TR) & (AR,PC)

1. (AC,IR,TR) & (INPR,PC,AR)

2. (AC,INPR,OUTR, IR) &(PC,AR,DR)

3. (AC, DR,IR, PC) & (AR,OUTR,TR)

Correct Answer(s):

- (AC,DR,TR) & (AR,PC)

________________________________________

## Question 31 (Marks: 1)

Question: What is the instruction cycle sequence?

Options:

0. Decode->Fetch->Read->Execute

1. Fetch->Decode->Read->Execute

2. Fetch->Execute->Read->Decode

3. None

Correct Answer(s):

- Fetch->Decode->Read->Execute

________________________________________

## Question 32 (Marks: 1)

Question: Which group of bits tells the computer which operation to perform?

Options:

0. Accumulator

1. Instruction code

2. PC Register

3. Instruction code and PC register

Correct Answer(s):

- Instruction code

________________________________________

## Question 33 (Marks: 1)

Question: Which part of a machine instruction specifies the operation to be performed?

Options:

0. Opcode

1. Operand

2. Address

3. Mode field

Correct Answer(s):

- Opcode

________________________________________

## Question 34 (Marks: 1)

Question: Which of the following is NOT typically included in an instruction format?

Options:

0. Opcode

1. Operand

2. Address

3. Data type

Correct Answer(s):

- Data type

________________________________________

## Question 35 (Marks: 1)

Question: What is the value of the following field in an register reference instruction format in basic computer. I-FLAG (BIT 15), OPCODE(BITS 12-14)

Options:

0. 0, 101 respectively

1. 0, 111 respectively

2. 1, 101 respectively

3. 1, 111 respectively

Correct Answer(s):

- 0, 111 respectively

________________________________________

## Question 36 (Marks: 1)

Question: An instruction cycle refers to

Options:

0. fetching an instruction

1. executing an instruction

2. fetching an instruction,decoding it, reading the contents of the effective addresses of the operands and executing the instruction

3. clock cycle time

Correct Answer(s):

- fetching an instruction,decoding it, reading the contents of the effective addresses of the operands and executing the instruction

________________________________________

## Question 37 (Marks: 1)

Question: In input-output &amp; interrupt system, what is the role of BSA and BUN instruction

Options:

0. BSA provides information about return address and interrupt subroutine location, BUN provides the indirect location to return to main program

1. BUN provides information about return address and interrupt subroutine location, BSA provides the indirect location to return to main program

2. BSA provides information about return address and interrupt subroutine location, BUN provides the direct location to return to main program

3. BUN provides information about return address and interrupt subroutine location, BSA provides the direct location to return to main program

Correct Answer(s):

- BSA provides information about return address and interrupt subroutine location, BUN provides the indirect location to return to main program

________________________________________

## Question 38 (Marks: 1)

Question: Which gate is called the “universal gate”?

Options:

0. OR

1. AND

2. NAND

3. NOT

Correct Answer(s):

- NAND

________________________________________

## Question 39 (Marks: 1)

Question: The identity A + 1 =

Options:

0. 1

1. A

2. 0

3. A’

Correct Answer(s):

- 1

________________________________________

## Question 40 (Marks: 1)

Question: A multiplexer is also known as:

Options:

0. Data selector

1. Data distributor

2. Encoder

3. Decoder

Correct Answer(s):

- Data selector

________________________________________

## Question 41 (Marks: 1)

Question: Flip-flop is a

Options:

0. Combinational circuit

1. Sequential circuit

2. Amplifier

3. Timer

Correct Answer(s):

- Sequential circuit

________________________________________

## Question 42 (Marks: 1)

Question: K-map is based on

Options:

0. Demorgan’s law

1. Boolean algebra

2. Probability

3. Set theory

Correct Answer(s):

- Boolean algebra

________________________________________

## Question 43 (Marks: 1)

Question: A 4-variable K-map has

Options:

0. 8 Cells

1. 12 Cells

2. 16 Cells

3. 32 Cells

Correct Answer(s):

- 16 Cells

________________________________________

## Question 44 (Marks: 1)

Question: F(A,B)=Σm(1,3) simplifies to

Options:

0. B

1. A

2. A’

3. B’

Correct Answer(s):

- B

________________________________________

## Question 45 (Marks: 1)

Question: The law A · A =

Options:

0. 0

1. 1

2. A

3. A’

Correct Answer(s):

- A

________________________________________

## Question 46 (Marks: 1)

Question: In K-map simplification, the don’t-care term can be treated as

Options:

0. 1 or 0

1. Always 1

2. Always 0

3. Ignored

Correct Answer(s):

- 1 or 0

________________________________________

## Question 47 (Marks: 1)

Question: A 3-to-8 decoder has how many inputs and outputs

Options:

0. 3 inputs, 8 outputs

1. 8 inputs, 3 outputs

2. 4 inputs, 16 outputs

3. 2 inputs, 4 outputs

Correct Answer(s):

- 3 inputs, 8 outputs

________________________________________

## Question 48 (Marks: 1)

Question: A 4×1 multiplexer has

Options:

0. 5select lines

1. 3 select lines

2. 4 select lines

3. 2 select lines

Correct Answer(s):

- 2 select lines

________________________________________

## Question 49 (Marks: 1)

Question: A + AB simplifies to

Options:

0. A

1. AB

2. B

3. A + B

Correct Answer(s):

- A

________________________________________

## Question 50 (Marks: 1)

Question: When both inputs of a JK flip-flop are HIGH, the output

Options:

0. Sets

1. Resets

2. Toggles

3. Remains same

Correct Answer(s):

- Toggles

________________________________________

## Question 51 (Marks: 1)

Question: The instruction code is a group of bits that specifies

Options:

0. An operation and operands

1. A register only

2. Data only

3. Memory size

Correct Answer(s):

- An operation and operands

________________________________________

## Question 52 (Marks: 1)

Question: The Accumulator (AC) is used to

Options:

0. Store intermediate results

1. Decode instructions

2. Hold memory addresses

3. Manage input-output

Correct Answer(s):

- Store intermediate results

________________________________________

## Question 53 (Marks: 1)

Question: The ALU performs

Options:

0. Control operations

1. I/O operations

2. Arithmetic and logical operations

3. Memory fetch

Correct Answer(s):

- Arithmetic and logical operations

________________________________________

## Question 54 (Marks: 1)

Question: CPU stands for

Options:

0. Central Processing Unit

1. Central Programming Unit

2. Control Processing Unit

3. Central Peripheral Unit

Correct Answer(s):

- Central Processing Unit

________________________________________

## Question 55 (Marks: 1)

Question: Input-output instructions communicate with

Options:

0. Memory

1. I/O devices

2. ALU

3. Register

Correct Answer(s):

- I/O devices

________________________________________

## Question 56 (Marks: 1)

Question: Stack pointer points to

Options:

0. First register

1. Bottom of memory

2. Program counter

3. Top of stack

Correct Answer(s):

- Top of stack

________________________________________

## Question 57 (Marks: 1)

Question: Micro-operation for PC increment

Options:

0. PC ← PC + 1

1. PC ← PC

2. PC ← IR

3. PC ← MAR

Correct Answer(s):

- PC ← PC + 1

________________________________________

## Question 58 (Marks: 1)

Question: The Timing and Control unit coordinates

Options:

0. Memory read only

1. Execution of micro-operations

2. ALU output

3. I/O operations only

Correct Answer(s):

- Execution of micro-operations

________________________________________

## Question 59 (Marks: 1)

Question: The micro-operations executed during an instruction cycle include

Options:

0. Fetch, Decode, Execute, Writeback

1. Only Fetch and Execute

2. Decode only

3. Read only

Correct Answer(s):

- Fetch, Decode, Execute, Writeback

________________________________________

## Question 60 (Marks: 1)

Question: The HLT instruction performs

Options:

0. Reset all registers

1. Clear memory

2. Stop CPU execution

3. Restart program

Correct Answer(s):

- Stop CPU execution

________________________________________

## Question 61 (Marks: 1)

Question: The Boolean expression for a 3-input AND gate is ________.

Options:

0. X = AB

1. X = ABC

2. X = A + B + C

3. X = AB + C

Correct Answer(s):

- X = ABC

________________________________________

## Question 62 (Marks: 1)

Question: What does the small bubble on the output of the NAND gate logic symbol mean?

Options:

0. open collector output

1. tristate

2. The output is inverted.

3. none of the above

Correct Answer(s):

- The output is inverted.

________________________________________

## Question 63 (Marks: 1)

Question: The output of an exclusive-OR gate is HIGH if

Options:

0. the inputs are unequal

1. all inputs are HIGH

2. all inputs are LOW

3. none of the above

Correct Answer(s):

- the inputs are unequal

________________________________________

## Question 64 (Marks: 1)

Question: How many input combinations would a truth table have for a six-input AND gate?

Options:

0. 32

1. 48

2. 64

3. 128

Correct Answer(s):

- 64

________________________________________

## Question 65 (Marks: 1)

Question: The inputs of a NAND gate are connected together. 

The resulting circuit is ………….

Options:

0. OR gate

1. AND gate

2. NOT gate

3. NOR gate

Correct Answer(s):

- NOT gate

________________________________________

## Question 66 (Marks: 1)

Question: Which is the correct order of sequence for representing the input values in K-map?

Options:

0. (00, 01, 10, 11)

1. (00, 10, 01, 11)

2. (00, 10, 11, 01)

3. (00, 01, 11, 10)

Correct Answer(s):

- (00, 01, 11, 10)

________________________________________

## Question 67 (Marks: 1)

Question: Which of the following is used to store binary data in digital systems?

Options:

0. Logic gates

1. Flip-flops

2. Multiplexers

3. Decoders

Correct Answer(s):

- Flip-flops

________________________________________

## Question 68 (Marks: 1)

Question: Which law states that A + A&#39; = 1?

Options:

0. Identity Law

1. Complement Law

2. Null Law

3. Distributive Law

Correct Answer(s):

- Complement Law

________________________________________

## Question 69 (Marks: 1)

Question: On a Karnaugh map, grouping the 0s produces __________

Options:

0. A POS expression

1. A SOP expression

2. AND-OR logic

3. NAND-NOR logic

Correct Answer(s):

- A POS expression

________________________________________

## Question 70 (Marks: 1)

Question: Evaluate the expression: (X + Z)(X + XZ’) + XY + Y.

Options:

0. XY+Z’

1. Y+XZ’+Y’Z

2. X’Z+Y

3. X+Y

Correct Answer(s):

- X+Y

________________________________________

## Question 71 (Marks: 1)

Question: For the minterm designation Y = ∑ m (1, 3, 5, 7) the complete expression is …………..

Options:

0. A'B'C+AB'C

1. A'B'C+AB'C+ABC+A'BC

2. A'B'C'+A'B'C+A'BC+AB'C

3. A'B'C'+ABC+A'B'C+AB'C

Correct Answer(s):

- A'B'C+AB'C+ABC+A'BC

________________________________________

## Question 72 (Marks: 1)

Question: D Flip Flop made from SR Flip Flop uses equation

Options:

0. S = D, R = D’

1. S = D’, R = D

2. S = 1, R = 0

3. S = 0, R = 1

Correct Answer(s):

- S = D, R = D’

________________________________________

## Question 73 (Marks: 1)

Question: Which register holds the address of the next instruction to be executed?

Options:

0. Program Counter

1. Instruction Register

2. Accumulator

3. Memory Address Register

Correct Answer(s):

- Program Counter

________________________________________

## Question 74 (Marks: 1)

Question: In a CPU, the instruction register (IR) holds the:

Options:

0. Address of the next instruction

1. Currently executing instruction

2. Data to be processed

3. Address of the data

Correct Answer(s):

- Currently executing instruction

________________________________________

## Question 75 (Marks: 1)

Question: The ALU makes use of _______ to store the intermediate results.

Options:

0. Accumulators

1. Registers

2. Heap

3. Stack

Correct Answer(s):

- Accumulators

________________________________________

## Question 76 (Marks: 1)

Question: The BSA instruction is______.

Options:

0. Branch and store accumulator

1. Branch and save return address

2. Branch and shift address

3. Branch and show accumulator

Correct Answer(s):

- Branch and save return address

________________________________________

## Question 77 (Marks: 1)

Question: Which instruction of the following is not included in register reference?

Options:

0. CLA

1. ADD

2. INC

3. HLT

Correct Answer(s):

- ADD

________________________________________

## Question 78 (Marks: 1)

Question: Which instruction format has &quot;7&quot; at MSB in hexadecimal representation of opcode?

Options:

0. Memory

1. Input-output

2. Register

3. None

Correct Answer(s):

- Register

________________________________________

## Question 79 (Marks: 1)

Question: In Interrupt cycle, generally the return value should be stored in which location?

Options:

0. PC + 1

1. 0

2. 1

3. PC

Correct Answer(s):

- 0

________________________________________

## Question 80 (Marks: 2)

Question: How many gates are required for F=XY+XZ+YZF?

Options:

0. 4

1. 5

2. 6

3. 7

Correct Answer(s):

- 5

________________________________________

## Question 81 (Marks: 2)

Question: Simplify the Boolean expression:[(A+B)’+(A’+B’)]’

Options:

0. A B + A’ B’

1. A ⊕ B

2. A + B

3. 0

Correct Answer(s):

- A B + A’ B’

________________________________________

## Question 82 (Marks: 2)

Question: Reduce the expression: [(AB)’+(A)’+(AB)]’

Options:

0. AB

1. 0

2. 1

3. A+B

Correct Answer(s):

- 0

________________________________________

## Question 83 (Marks: 2)

Question: Which of the following expressions is the complement of  F(x, y, z) = Σ(1, 3, 5, 7)?

Options:

0. F'(x, y, z) = Σ(0, 2, 4, 6)

1. F'(x, y, z) = Π(0, 2, 4, 6)

2. F'(x, y, z) = Σ(2, 4, 6, 8)

3. F'(x, y, z) = Π(1, 3, 5, 7)

Correct Answer(s):

- F'(x, y, z) = Π(0, 2, 4, 6)

________________________________________

## Question 84 (Marks: 2)

Question: If the present output Qn = 1 and inputs are J = 1, K = 1, then Qn+1 = ______.

Options:

0. 0

1. 1

2. Undefined

3. No change

Correct Answer(s):

- 0

________________________________________

## Question 85 (Marks: 2)

Question: How many gates for F=XY+X(X′+Z)+Y′Z?

Options:

0. 6

1. 7

2. 8

3. 9

Correct Answer(s):

- 8

________________________________________

## Question 86 (Marks: 2)

Question: Simplify the expression F(A,B,C) = Σ (1,3,5,6,7).

Options:

0. AC’+B’

1. AB+C

2. AB’+B’C’

3. A’BC+B’C+AC

Correct Answer(s):

- AB+C

________________________________________

## Question 87 (Marks: 2)

Question: Simplify Σm(9,10,12)+d(3,5,6,7,11,13,14,15)

Options:

0. AB+CD+BC

1. ABC+ABD

2. A(B+C+D)

3. AB+BC+AD

Correct Answer(s):

- A(B+C+D)

________________________________________

## Question 88 (Marks: 2)

Question: The boolean function Y= AB+CD is to be realized using only 2-input NAND gates. The minimum number of the gates required is

Options:

0. 2

1. 3

2. 4

3. 5

Correct Answer(s):

- 3

________________________________________

## Question 89 (Marks: 2)

Question: Simplify Σm(5,6,7,9,10,11,13,14,15).

Options:

0. A+B

1. (A+B)(C+D)

2. AB

3. CD

Correct Answer(s):

- (A+B)(C+D)

________________________________________

## Question 90 (Marks: 2)

Question: Simplify ΠM(3,6,8,11,13,14).d(1,5,7,10).

Options:

0. (B+C’+D’)(A’+B+D)(B’+C+D’)(B’+C’+D)

1. (A+B)(C+D)

2. (A’+B’)(B+C’)(C’+D)

3. (A+B’)(C’+D’)

Correct Answer(s):

- (B+C’+D’)(A’+B+D)(B’+C+D’)(B’+C’+D)

________________________________________

## Question 91 (Marks: 2)

Question: If J = 0 and K = 0 for a JK flip-flop, then Qn+1=_______.

Options:

0. 0

1. 1

2. Toggle

3. same as Qn

Correct Answer(s):

- same as Qn

________________________________________

## Question 92 (Marks: 2)

Question: Which of the following logical operations is represented by the + sign in Boolean algebra?

Options:

0. inversion

1. AND

2. OR

3. complementation

Correct Answer(s):

- OR

________________________________________

## Question 93 (Marks: 2)

Question: The NOR gate is equivalent to which logic operation according to De Morgan’s theorem?

Options:

0. (A + B)’ = A’.B’

1. (A.B)’ = A’ + B’

2. A’B

3. A + B

Correct Answer(s):

- (A + B)’ = A’.B’

________________________________________

## Question 94 (Marks: 2)

Question: How many gates would be required to implement XY + X(X + Z) + Y(X + Z) before simplification?

Options:

0. 1

1. 2

2. 4

3. 5

Correct Answer(s):

- 5

________________________________________

## Question 95 (Marks: 2)

Question: One of De Morgan’s theorems: (A+B)′=A′B′. This means there is no logical difference between ______ and ______.

Options:

0. a NOR and an AND gate with inverted inputs

1. a NAND and an OR gate with inverted inputs

2. an AND and a NOR gate with inverted inputs

3. a NOR and a NAND gate with inverted inputs

Correct Answer(s):

- a NOR and an AND gate with inverted inputs

________________________________________

## Question 96 (Marks: 2)

Question: For a computer with 64 distinct operations, bits required for the opcode = ______.

Options:

0. 6

1. 8

2. 32

3. 16

Correct Answer(s):

- 6

________________________________________

## Question 97 (Marks: 2)

Question: A memory unit with 256 K words of 32 bits: data input bits = ______; address input bits = ______.

Options:

0. 32, 18

1. 18, 32

2. 8, 8

3. 32, 32

Correct Answer(s):

- 32, 18

________________________________________

## Question 98 (Marks: 2)

Question: What is the size (in bits) of the IR register in 4096×16 word memory?

Options:

0. 0-11 bits

1. 16

2. 8

3. 0-15 bits

Correct Answer(s):

- 16

________________________________________

## Question 99 (Marks: 2)

Question: In input/output reference instruction format (basic computer), I-FLAG (bit 15), OPCODE (bits 12–14); what is the value of this field?

Options:

0. 0, 101 respectively

1. 0, 111 respectively

2. 1, 101 respectively

3. 1, 111 respectively

Correct Answer(s):

- 1, 111 respectively

________________________________________

## Question 100 (Marks: 2)

Question: What does a computer bus line consist of?

Options:

0. Accumulator

1. Flip Flops

2. Set of parallel lines

3. Registers

Correct Answer(s):

- Set of parallel lines

________________________________________

## Question 101 (Marks: 2)

Question: The key hardware elements of hardwired control unit

Options:

0. Sequence counter

1. 3x8 decoder & 4x16 decoder

2. Control logic arrays

3. All of these

Correct Answer(s):

- All of these

________________________________________

## Question 102 (Marks: 2)

Question: In the 2048*16 instruction format, what is the maximum addressable memory size?

Options:

0. 16 KB

1. 8 KB

2. 4 KB

3. 2 KB

Correct Answer(s):

- 2 KB

________________________________________

## Question 103 (Marks: 2)

Question: A logic gate with two inputs and one output is

Options:

0. Decoder

1. Flip-flop

2. Basic logic gate

3. Multiplexer

Correct Answer(s):

- Basic logic gate

________________________________________

## Question 104 (Marks: 2)

Question: The edge-triggered flip-flop responds to

Options:

0. Change in input

1. Level of clock

2. Transition of clock

3. Data change

Correct Answer(s):

- Transition of clock

________________________________________

## Question 105 (Marks: 2)

Question: POS stands for

Options:

0. Product of Sums

1. Product of Subtraction

2. Sum of Products

3. Power of Sums

Correct Answer(s):

- Product of Sums

________________________________________

## Question 106 (Marks: 2)

Question: A 7-segment display is driven by

Options:

0. Decoder

1. Encoder

2. Flip-flop

3. Counter

Correct Answer(s):

- Decoder

________________________________________

## Question 107 (Marks: 2)

Question: Simplify F(A,B,C)=Σm(1,3,5,7)

Options:

0. C

1. A

2. A + C

3. B + C

Correct Answer(s):

- C

________________________________________

## Question 108 (Marks: 2)

Question: Simplify F(A,B,C,D)=Σm(4,5,6,7,12,13,14,15)

Options:

0. B

1. A

2. C

3. D

Correct Answer(s):

- B

________________________________________

## Question 109 (Marks: 2)

Question: F(A,B,C,D)=Σm(0,1,8,9) simplifies to

Options:

0. A’B’

1. B’C’

2. A’C’

3. A + B

Correct Answer(s):

- B’C’

________________________________________

## Question 110 (Marks: 2)

Question: A combinational circuit that converts binary data from one form to another is

Options:

0. Decoder

1. Multiplexer

2. Counter

3. Latch

Correct Answer(s):

- Decoder

________________________________________

## Question 111 (Marks: 2)

Question: minimal expression for F(A,B,C) = Σ(0,2,6,7) is

Options:

0. A’C’ + AB

1. A + BC

2. A’B + AB’

3. AB + BC

Correct Answer(s):

- A’C’ + AB

________________________________________

## Question 112 (Marks: 2)

Question: Instruction set completeness requires ability to

Options:

0. Perform arithmetic, logic, transfer, control operations

1. Perform only I/O

2. Only control

3. Only arithmetic

Correct Answer(s):

- Perform arithmetic, logic, transfer, control operations

________________________________________

## Question 113 (Marks: 2)

Question: The instruction ADD R1, R2 belongs to which instruction type

Options:

0. Control transfer

1. Arithmetic

2. Logical

3. I/O

Correct Answer(s):

- Arithmetic

________________________________________

## Question 114 (Marks: 2)

Question: The ADD R1, R2 instruction performs

Options:

0. R2 ← R1 + R2

1. R1 ← R1 + R2

2. R1 ← R2

3. R2 ← R1

Correct Answer(s):

- R1 ← R1 + R2

________________________________________

## Question 115 (Marks: 2)

Question: The increment instruction (INC A) affects

Options:

0. Only the accumulator (A ← A + 1)

1. Flags only

2. Memory address

3. Program counter

Correct Answer(s):

- Only the accumulator (A ← A + 1)

________________________________________

## Question 116 (Marks: 2)

Question: Complement of the expression A’B + CD’ is ________

Options:

0. (A’ + B)(C’ + D)

1. (A + B’)(C’ + D)

2. (A’ + B)(C’ + D')

3. (A + B’)(C + D’)

Correct Answer(s):

- (A + B’)(C’ + D)

________________________________________

## Question 117 (Marks: 2)

Question: Use Boolean algebra to find the most simplified SOP 

expression for F = ABD + CD + ACD + ABC + ABCD.

Options:

0. F = ABD + ABC + CD

1. F = CD + AD

2. F = BC + AB

3. F = AC + AD

Correct Answer(s):

- F = ABD + ABC + CD

________________________________________

## Question 118 (Marks: 2)

Question: When both set and reset are disabled in S-R flip flop then the output will be __________

Options:

0. Set

1. Reset

2. No change

3. Indeterminate

Correct Answer(s):

- No change

________________________________________

## Question 119 (Marks: 2)

Question: When S=0, R=0, CLK=X then the output will be ___________

Options:

0. No change

1. Set

2. Reset

3. Invalid

Correct Answer(s):

- No change

________________________________________

## Question 120 (Marks: 2)

Question: How many NAND gates does the D flip flop circuit consists of?

Options:

0. One

1. Two

2. Three

3. Four

Correct Answer(s):

- Four

________________________________________

## Question 121 (Marks: 2)

Question: Number of select lines would be required for an 8-line-to-1-line multiplexer?

Options:

0. 2

1. 4

2. 8

3. 3

Correct Answer(s):

- 3

________________________________________

## Question 122 (Marks: 2)

Question: Find Product of Sum (POS) form of f (a, b, c, d) = πM (1, 4, 6, 9)

Options:

0. (d + a + b'). (b’ + c’ + d')

1. (d + a + b'). (b + c + d')

2. (d + a + b'). (b’ + c + d)

3. (d + a + b'). (b + c’ + d')

Correct Answer(s):

- (d + a + b'). (b + c + d')

________________________________________

## Question 123 (Marks: 2)

Question: ________ are the different type/s of generating control signals.

Options:

0. Hardwired

1. Micro-instruction

2. Micro-programmed

3. Both Micro-programmed and Hardwired

Correct Answer(s):

- Both Micro-programmed and Hardwired

________________________________________

## Question 124 (Marks: 2)

Question: How does a hardwired control unit differ from a microprogrammed control unit?

Options:

0. Hardwired is slower and more flexible

1. Microprogrammed is faster but less flexible

2. Both are the same

3. Hardwired is faster and less flexible

Correct Answer(s):

- Hardwired is faster and less flexible

________________________________________

## Question 125 (Marks: 2)

Question: Which type of decoder is used to decode operation code of instruction register in control unit?

Options:

0. 2 x 4

1. 4 x 16

2. 5 x 32

3. 3 x 8

Correct Answer(s):

- 3 x 8

________________________________________

## Question 126 (Marks: 2)

Question: A memory-reference instruction is identified (in basic computer) when the instruction register bits are:

Options:

0. I=0 & IR(14-12) = 111

1. I=1 & IR(14-12) = 111

2. IR(14-12) ≠ 111

3. NONE OF ABOVE

Correct Answer(s):

- IR(14-12) ≠ 111

________________________________________

## Question 127 (Marks: 2)

Question: Control function for SKI (Skip if input flag) belongs to:

Options:

0. memory reference group

1. register reference group

2. I/O reference group

3. ALU group

Correct Answer(s):

- I/O reference group

________________________________________